Haghtanak SS

| Company | Type | Direction | Line | Frequency | Note |
| --- | --- | --- | --- | --- | --- |
| Siemens | ESB 2000 | Haghtanak-Shahumyan-2 | <<Haghtanak>> phase. ‘’C’’ 220kV | 244-252/252-260 |  |
| Siemens | ESB 2000 | Haghtanak- Ararat-2 | <<Erebuni>> phase. ‘’C’’ 220kV | 200-208/208-216 | As per reconstruction project of Yerevan TPP the line should be disconnected and implemented the entrance to Yerevan TPP |

Ashnak SS

| Company | Type | Direction | Line | Frequency | Note |
| --- | --- | --- | --- | --- | --- |
| Siemens | ESB 2000 | Ashnak- ARM NPP | <<Ashnak-1>> phase. ‘’C'’ 220kV | 152-160/160-164 | Heavy noise from Turkey |
| Siemens | ESB 2000 | Ashnak- ARM NPP | <<Ashnak-2>> phase. ‘’C’' 220kV | 352-360/360-368 |  |
| ABB | ETL-600 | Ashnak- Gyumry-2 | <<Ani>> phase. ‘’C’' 220kV | 312-320/320-328 |  |
| Siemens | Power-Link | Armavir- Ashnak | <<Igdir>> phase. ‘’C’' 110kV | 184-188/188-192 |  |

Charentsavan-3 SS

| Company | Type | Direction | Line | Frequency | Note |
| --- | --- | --- | --- | --- | --- |
| Neptun | ABC-3 | Hrazdan HPP- Charentsavan-3 | <<Solak>> phase. ‘’B'’ 110kV | 980-992/920-932 | To be dismantled |

Vanadzor-1 SS

There are no any HF links existing.